RESEARCH ROADMAP

1-D Axial, Non-Isothermal

Adsorption Breakthrough Modelling

in a Fixed-Bed Column

A literature-review companion for the single-component CO₂ / N₂ (carrier) system:

isotherm & temperature-dependent mechanisms · linear driving force kinetics · method-of-lines numerical integration

Prepared for

Loh John Ray

1 July 2026

Source library: 13 papers & textbook chapters + 1 technical reference

# Contents

# How to Use This Roadmap

This document turns your working folder of papers and textbook chapters into a navigable research tool. Sources are grouped under three pillars that mirror your review's structure, and each source is presented as a colour-coded card summarising its contribution, its key equations or findings, and why it matters for a 1-D axial, non-isothermal breakthrough model of CO₂ in N₂ on a fixed bed.

Every card ends with a ruled space for your own notes and a status checklist, so this can double as your working log as you move through the pile.

## Colour legend — study type

| TEXTBOOK | Foundational chapters from standard references — canonical derivations and definitions you can cite with confidence. |
| --- | --- |
| REVIEW | Survey papers that classify and compare many models — use these to justify model choice and terminology. |
| ANALYTICAL MODEL | Closed-form or semi-analytical solutions — useful as benchmarks / limiting cases for your numerical model. |
| NUMERICAL / CFD | Papers whose core result is a numerical (CFD or ODE/PDE) solution, validated against experimental data. |
| TECHNICAL REF. | External technical documentation (not a research paper) — implementation-level guidance. |

## Reading each card

- Header strip — pillar code (e.g. A1), study-type tag, and the accent colour used throughout for that source type.

- Title & citation — full reference, so the card is self-contained even away from your reference manager.

- Key contribution bullets — the specific mechanisms, equations, or results relevant to this project, pulled directly from the text.

- Relevance line — a one-sentence bridge from the source to your own model.

- Notes area — blank ruled lines plus a read / extracted / cited checklist.

# The Three Pillars

Your review brief breaks into three linked questions. Each becomes a pillar of this roadmap; together they cover the physics, the kinetics, and the computation needed for a defensible 1-D model.

PILLAR A Mechanisms: CO₂/N₂, Isotherms & Temperature-Dependent Parameters How single-component CO₂ adsorption on a solid, in an inert N₂ carrier, is linked mathematically to an equilibrium isotherm whose parameters (affinity constants, saturation capacities, isosteric heat) vary with temperature — the coupling that makes the model ‘non-isothermal’.

PILLAR B Evidence for Linear Driving Force (LDF) Kinetics Why the lumped LDF rate law dq/dt = k(q*−q) is used almost universally in place of a full intraparticle diffusion model, what k = 15D/rᵨ² approximates, and where the approximation is validated — or breaks down — against exact and experimental solutions.

PILLAR C Numerical Integration: Method of Lines + solve_ivp How the coupled hyperbolic/parabolic PDE system (mass, energy, and LDF kinetics) is reduced to a large stiff ODE system via spatial discretisation (Method of Lines), and why implicit integrators (BDF, LSODA) in scipy.integrate.solve_ivp are the appropriate tool.

# Pillar A — Mechanisms: Isotherms & Temperature-Dependent Parameters

These sources establish how the equilibrium isotherm links gas-phase CO₂ concentration to adsorbed-phase loading, and — critically for a non-isothermal model — how the isotherm's own parameters and the heat of adsorption change with temperature.

| A1 NUMERICAL / CFD | CFD-Based Model of Adsorption Columns: Validation Fabian Ramos, H.S., Baliga, C., Rajendran, A. & Nikrityuk, P.A. (2024). Chemical Engineering Science, 285, 119606. - Couples continuity, momentum, species and energy transport equations for CO₂/N₂ on zeolite 13X, implemented as user-defined functions in ANSYS Fluent and validated against published single- and dual-component breakthrough experiments. - Equilibrium is modelled with a Dual-Site Langmuir (DSL) isotherm: q* = q₁bC/(1+ΣbC) + q₂dC/(1+ΣdC), with the affinity constants bᵢ and dᵢ following Arrhenius-type expressions bᵢ = b₀,ᵢ·exp(−ΔU_{b,i}/RT) (their Eqs. 15–16) — the explicit temperature-dependent parameter link your brief asks about. - The isosteric heat of adsorption is treated as loading- and temperature-dependent (linear in T, plus an empirical loading correction for CO₂, their Eq. 10), and enters the energy balance as the adsorption heat-release term. - The adsorption rate itself is closed with an LDF expression, k_LDF = 15D_eff/r_p², giving a directly transferable set of equations (Eqs. 1–9, 11–16) for a 1-D reduction of the same physics. Relevance to this model: The single richest source in your library for a fully worked, validated CO₂/N₂ mechanism: DSL isotherm + Arrhenius T-dependence + LDF + isosteric heat, all in one internally consistent model you can cite equation-by-equation. |
| --- | --- |

Theme: A (primary) · also B, C	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| A2 REVIEW | Guidelines for the Use and Interpretation of Adsorption Isotherm Models: A Review Al-Ghouti, M.A. & Da'ana, D.A. (2020). Journal of Hazardous Materials, 393, 122383. - Systematic guide to two-, three- and four-parameter isotherms (Langmuir, Freundlich, Sips, Toth, Redlich–Peterson, Dubinin–Radushkevich, BET, Khan, Fritz–Schlünder and others), with worked linear and non-linear forms. - Flags which parameters are explicitly temperature-dependent (e.g. Freundlich K_F and 1/n both vary with T; Temkin's b_T is tied to the heat of sorption) — useful for deciding which parameters in your own isotherm need an explicit T(z,t) dependence. - Discusses correct model-selection practice (error-function comparison, avoiding over-fitting with too many parameters) — directly applicable when you justify choosing DSL/Toth over simple Langmuir for CO₂/N₂. Relevance to this model: Use as the methodological backbone for the isotherm-selection section of your model write-up: it gives you the vocabulary and validation criteria to defend whichever isotherm you adopt. |
| --- | --- |

Theme: A (primary)	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| A3 ANALYTICAL MODEL | Concentration and Temperature Profiles in a Fixed-Bed Column Based on an Analytical Solution of the Axial Dispersion Model for Binary and Multicomponent Non-Isothermal Adsorption Processes Danilov, V.A., De Schepper, P., Cousin-Saint-Remi, J. & Denayer, J.F.M. (2019). Computers & Chemical Engineering, 122, 41–52. - Builds the coupled non-isothermal axial-dispersion model in terms of four field variables: component molar flux F, adsorbate loading q, gas-phase temperature Tᴳ and solid-phase temperature Tˢ — a template state vector for your own 1-D formulation. - Uses an LDF kinetic closure and a Langmuir isotherm, with the heat of adsorption ΔH_ads entering the heat-balance source term directly. - Derives an analytical simplification for the adsorption driving force ψ by exploiting the relationship ∂y/∂t = (∂y/∂q)(∂q/∂t), then closes the remaining coupled PDEs by numerical integration — i.e. an explicit semi-analytical + numerical hybrid. - Validated for both binary and multicomponent non-isothermal breakthrough. Relevance to this model: The closest published analogue to your target model. Use its governing-equation set (Table 1 in the paper) as a cross-check for your own mass/energy balance formulation, and its results as a benchmark case. |
| --- | --- |

Theme: A (primary) · also B	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

# Pillar B — Evidence for Linear Driving Force (LDF) Kinetics

These sources trace the LDF approximation from its original derivation through independent validations against exact diffusion solutions, alternative kinetic orders, and real breakthrough data — giving you a defensible basis for adopting dq/dt = k(q*−q) rather than a full pore-diffusion model.

| B1 TEXTBOOK | Principles of Adsorption and Adsorption Processes — Ch. 6 (Kinetics of Sorption in Batch Systems) & Ch. 8 (Dynamics of Adsorption Columns: Single-Transition Systems) Ruthven, D.M. (1984). John Wiley & Sons, New York. - Gives the canonical derivation of the LDF approximation, attributing it to Glueckauf (1955): a linear rate law dq/dt = k(q*−q) with k = 15D/r² reproduces the exact Rosen (diffusion) solution for a linear isotherm, for a wide range of boundary conditions (§8.5). - Shows via moments analysis (van der Laan's theorem) how axial dispersion and multiple mass-transfer resistances combine additively into a single effective rate coefficient k′ (Eq. 8.42), extending the basic Glueckauf result. - States explicitly that the LDF/diffusion equivalence ‘works well for linear and nearly linear systems’ but ‘breaks down when the isotherm is highly nonlinear’ (§8.6) — an important caveat for a favourable-isotherm system like CO₂ on zeolite. - Chapter 6 separately analyses non-isothermal single-particle kinetics: when heat-transfer resistance is significant, uptake is governed by two parameters (α′, β) spanning pure diffusion control to pure heat-transfer control (Eqs. 6.62–6.70). - Chapter 8 also derives the isothermal governing PDE pair (fluid mass balance + particle/LDF rate expression, Eqs. 8.1–8.2) and shows how a favourable isotherm produces ‘constant-pattern’ (self-sharpening) fronts, while an unfavourable one produces continuously spreading (‘proportionate-pattern’) fronts. Relevance to this model: Your primary citation for the LDF rate law itself, its rate-constant formula, its range of validity, and the qualitative front behaviour (constant- vs proportionate-pattern) your simulated breakthrough curves should reproduce. |
| --- | --- |

Theme: B (primary) · also A	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| B2 REVIEW | Review: Mathematically Modeling Fixed-Bed Adsorption in Aqueous Systems Xu, Z., Cai, J. & Pan, B. (2013). Journal of Zhejiang University-SCIENCE A, 14(3), 155–176. - Places the LDF model (dq/dt = k_e(q_s−q_a), their Eq. 22, citing Glueckauf 1955) within a full taxonomy: general rate models, wave-propagation theory, constant-pattern model, Clark, Thomas, Bohart–Adams, Yoon–Nelson, Wang, Wolborska and modified dose–response models. - Discusses the mechanistic assumption behind LDF — a single lumped coefficient standing in for the true intrapellet (macropore + micropore) diffusion — and its main shortcoming: loss of resolution when diffusional resistances are not well separated in time-scale. - Useful cross-reference table of which models require numerical vs analytical solution, and their typical parameter-fitting routes. Relevance to this model: Positions LDF relative to every other common breakthrough model, so you can justify — in one paragraph — why LDF rather than, say, a Thomas or general rate model is the right level of complexity for a fixed-bed CO₂/N₂ study. |
| --- | --- |

Theme: B (primary) · also A	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| B3 ANALYTICAL MODEL | Theoretical Prediction of Breakthrough Curves for Molecular Sieve Adsorption Columns — I. Asymptotic Solutions Garg, D.R. & Ruthven, D.M. (1973). Chemical Engineering Science, 28(3), 791–798. - Derives asymptotic (‘constant-pattern’) breakthrough-curve solutions for micropore- and macropore-diffusion-controlled kinetics under a Langmuir isotherm, for isothermal single-component adsorption from a lean gas. - Because these are exact diffusion-control solutions, they are the natural benchmark against which an LDF constant-pattern solution (k = 15D/r²) should be checked for a favourable isotherm. - Restricted to long columns / lean gas — note this boundary-condition simplification when comparing against a finite-length numerical model. Relevance to this model: Use as an analytical validation case: run your LDF-based numerical model to the constant-pattern limit and confirm it converges to these asymptotic diffusion-controlled curves within the expected error. |
| --- | --- |

Theme: B (primary)	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| B4 ANALYTICAL MODEL | Perturbative Solutions for Non-Linear Fixed-Bed Adsorption — I. Approximate Analytical Solutions for Asymptotic Fronts Gorius, A., Bailly, M. & Tondeur, D. (1991). Chemical Engineering Science, 46(2), 677–684. - General non-linear chromatography model (axial dispersion + a generic mass-transfer law, of which LDF is a special case) solved by perturbation analysis rather than by direct numerical integration. - Formally justifies the widely used but often un-derived assumption that the ‘heights of transfer units’ (HTU) for axial dispersion and for mass-transfer resistance are additive, and derives the applicability criterion for when this holds. - Validates its perturbative fronts directly against full numerical integration of the PDE system — the paper explicitly compares analytical, numerical, and experimental results. Relevance to this model: Gives you a second, independent theoretical route to sanity-check a numerical solver: if your Method-of-Lines solution and this perturbative solution disagree outside the stated applicability criterion, that flags a genuine modelling (not numerical) issue. |
| --- | --- |

Theme: B (primary)	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| B5 ANALYTICAL MODEL | Column Dynamics for Multicomponent Adsorption — Constant Pattern Formation Kumar, R. (1986). Separation Science and Technology, 21(10), 1039–1046. - Extends constant-pattern theory from single-component to multicomponent adsorption, examining how competing adsorbates' fronts interact and separate. - Directly relevant if CO₂/N₂ is later extended to a genuinely competitive two-component system rather than treating N₂ purely as inert carrier. Relevance to this model: Keep in reserve for a future extension of the model to competitive multicomponent adsorption; not essential for a single-adsorbable-component (CO₂ in inert N₂) baseline model. |
| --- | --- |

Theme: B (primary)	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| B6 ANALYTICAL MODEL | Analytical Expressions for the Adsorbate Breakthrough Curve from a Fixed Bed of Adsorbent with First-Order and Second-Order Kinetic Models Skrip, S., Das, G.K. & Chatterjee, S.G. (2013). Indian Chemical Engineer, 55(2), 87–103. - Derives closed-form breakthrough expressions under first-order and second-order kinetic assumptions — an alternative to the linear (first-order-in-driving-force) LDF assumption. - Provides a fast, non-numerical way to test the sensitivity of predicted breakthrough curves to the assumed kinetic order. Relevance to this model: A convenient sensitivity check: if reviewers question the LDF assumption, these closed forms let you show how much the predicted curve shape would change under a different kinetic order, without rebuilding the full numerical model. |
| --- | --- |

Theme: B (primary)	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| B7 ANALYTICAL MODEL | Modeling Breakthrough Curves in a Fixed Bed of Activated Carbon Fiber — Exact Solution and Parabolic Approximation Xiu, G. (1996). Chemical Engineering Science, 51(16), 4039–4041. - Derives an exact Laplace-domain solution for breakthrough curves accounting for axial dispersion, external film mass transfer and intraparticle pore diffusion, plus a simpler parabolic-profile (LDF-like) approximation. - Quantifies the discrepancy introduced by the parabolic/lumped simplification relative to the exact solution, as a function of four dimensionless groups. Relevance to this model: A second, independent quantitative error bound (beyond Garg & Ruthven 1973) for how far an LDF-style lumped approximation can drift from an exact diffusion solution — useful for stating uncertainty on your kinetic assumption. |
| --- | --- |

Theme: B (primary)	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| B8 ANALYTICAL MODEL | Mass Transfer from a Fluid Flowing through a Porous Media Myers, T.G. & Font, F. (2020). International Journal of Heat and Mass Transfer, 163, 120374. - Develops a mass-transfer model for a fluid flowing through a packed column, non-dimensionalises the governing equations to show which terms may be dropped, and obtains an analytical travelling-wave solution for the breakthrough curve. - Explicitly fits an LDF-type model to experimental CO₂ / activated-carbon breakthrough data and reports ‘an excellent fit’, providing direct experimental support for the LDF assumption in a CO₂-relevant system. - Also validates the reduced model against contaminant removal data (amoxicillin, Congo red dye) beyond gas-phase carbon capture, showing the LDF/travelling-wave approach generalises across systems. Relevance to this model: Your strongest piece of experimental evidence that LDF kinetics quantitatively describe real CO₂ breakthrough behaviour, not just a mathematically convenient simplification. |
| --- | --- |

Theme: B (primary) · also A	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

↳  See also A1 (Fabian Ramos et al., 2024): its validated LDF + Arrhenius closure, checked against real breakthrough experiments on zeolite 13X, is additional numerical/CFD-level evidence for the LDF assumption specifically for CO₂/N₂.

# Pillar C — Numerical Integration: Method of Lines + solve_ivp

None of the papers in your library discuss scipy by name — that is expected, since they are the physics/mathematics literature, not software documentation. This pillar pairs the mathematical foundations available locally (characteristics, hyperbolic systems) with the practical scipy reference you'll need to implement the solver.

| C1 TEXTBOOK | Finite Volume Methods for Hyperbolic Problems — Ch. 6 (Characteristics) & Ch. 7 (The Riemann Problem for Nonlinear Systems) LeVeque, R.J. Cambridge University Press. - Rigorous treatment of characteristic variables, simple waves, the wave equation, and linearisation of nonlinear hyperbolic systems (§6.1–6.4). - Covers the Riemann problem, genuine nonlinearity, the Lax entropy condition and linear degeneracy (§7.1–7.6) — the formal explanation for why favourable-isotherm adsorption fronts sharpen into shock-like, self-similar (‘constant-pattern’) profiles, matching the qualitative picture in Ruthven Ch. 8. - Establishes why naive explicit, low-order spatial discretisations of a convection-dominated PDE smear or oscillate around such fronts — the underlying reason a Method-of-Lines scheme needs either upwind/flux-limited spatial differencing or enough grid resolution before time integration. Relevance to this model: Justifies the spatial-discretisation choice you make before applying solve_ivp: because the concentration and temperature fronts are shock-like, the axial (z) discretisation must be upwind-biased or otherwise stabilised, or the resulting ODE system will be poorly conditioned regardless of which time integrator is chosen. |
| --- | --- |

Theme: C (primary)	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| C2 TEXTBOOK | Partial Differential Equations — Ch. 3 (Nonlinear First-Order PDE: Characteristics) Evans, L.C. American Mathematical Society, Graduate Studies in Mathematics. - Formal method-of-characteristics theory for first-order (hyperbolic) PDEs, including existence/uniqueness conditions and the treatment of characteristic curves that cross (shock formation). - Mathematically underwrites the ‘kinematic wave’ description of the concentration front that Ruthven derives heuristically (Ch. 8, Eq. 8.10–8.11): both describe the same characteristic-line propagation, one rigorously and one physically. Relevance to this model: A rigor check rather than a numerical recipe: use it to confirm the hyperbolic (convective) part of your governing PDE system is well-posed before you discretise and hand it to a stiff ODE solver. |
| --- | --- |

Theme: C (primary)	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

| C3 TECHNICAL REF. | scipy.integrate.solve_ivp — SciPy Manual SciPy Developers. docs.scipy.org (v1.18, accessed July 2026). - Explicit vs implicit choice: RK23/RK45/DOP853 are for non-stiff problems; Radau and BDF are implicit methods for stiff problems; LSODA auto-switches between an Adams (non-stiff) and a BDF (stiff) method and can ‘also be a good universal choice’ when stiffness is uncertain. - BDF is an implicit, variable-order (1–5) multistep method using a backward-differentiation formula with a quasi-constant step scheme (NDF-enhanced accuracy). - The Jacobian (required by Radau, BDF and LSODA) is approximated by finite differences if not supplied — but the documentation explicitly recommends supplying it for performance; for LSODA a banded Jacobian can be passed via lband/uband. - rtol/atol default to 1e-3 / 1e-6; for a stiff, physically stiff system such as a fixed-bed breakthrough model with fast LDF kinetics and sharp fronts, tighter tolerances are typically needed for a converged front shape. Relevance to this model: The direct implementation reference for the time-integration half of your Method-of-Lines solver: after discretising ∂c/∂z, ∂²c/∂z², ∂T/∂z etc. into a right-hand-side function f(t,y), this documents exactly how to call solve_ivp(f, t_span, y0, method='BDF'¦'LSODA', jac=..., rtol=..., atol=...). |
| --- | --- |

Theme: C (primary)	Status:  ☐ Read   ☐ Equations extracted   ☐ Cited in draft

Your notes

↳  See also A3 (Danilov et al., 2019): explicitly closes its coupled non-isothermal PDE system by numerical integration after an analytical simplification — a worked example of the semi-analytical + numerical hybrid you may end up using.

↳  See also A1 (Fabian Ramos et al., 2024): solves the same class of physics with a full CFD (finite-volume, implicit pressure-based) solver rather than a 1-D Method-of-Lines/solve_ivp approach — useful as a higher-fidelity point of comparison once your 1-D model is validated.

## Why the fixed-bed problem is numerically stiff

- Fast local kinetics vs. slow bulk transport: the LDF rate constant k (or k_LDF = 15D_eff/r_p²) is typically orders of magnitude faster than the convective residence time L/u, producing a large spread of eigenvalues in the discretised system — the textbook definition of stiffness.

- Sharp, self-sharpening fronts: as Pillar B and C1 show, a favourable isotherm drives the concentration (and coupled temperature) profile toward a constant-pattern, shock-like front; resolving it needs many spatial nodes, which multiplies the number of coupled ODEs and tightens the stability constraint on any explicit integrator.

- Nonlinear, temperature-coupled isotherm: Arrhenius-type T-dependence in b_i, d_i (Pillar A) couples the mass and energy balances nonlinearly, further widening the eigenvalue spread near the thermal front.

- Practical consequence: explicit methods (RK45) will need impractically small time-steps or fail outright (step-size underflow); BDF or LSODA with an appropriately sparse/banded Jacobian is the standard, defensible choice — consistent with how Danilov et al. (2019) and Fabian Ramos et al. (2024) both resort to numerical (not purely analytical) integration once temperature coupling is introduced.

# Consolidated Governing-Equation Reference Sheet

A synthesis of the equations extracted across Pillars A–C, in one consistent notation, for direct use when you draft your own model. Symbols: cᵢ gas-phase concentration, qᵢ solid-phase loading, qᵢ* equilibrium loading, T gas/solid temperature (pseudo-homogeneous), z axial coordinate, ε bed voidage, u interstitial velocity, D_L axial dispersion coefficient, ρ_p particle density.

### 1. Fluid-phase mass balance (per adsorbable species i)

| Quantity | Expression | Notes |
| --- | --- | --- |
| Mass balance | ε∂cᵢ/∂t + u∂cᵢ/∂z = εD_L∂²cᵢ/∂z² − (1−ε)ρ_p∂qᵢ/∂t | Ruthven Eq. 8.1; Fabian Ramos Eq. 4 |

### 2. LDF kinetics (Pillar B)

| Quantity | Expression | Notes |
| --- | --- | --- |
| Rate law | dqᵢ/dt = k_i(qᵢ* − qᵢ) | Glueckauf (1955); Ruthven §8.5 |
| Rate constant | k_i = 15D_eff,i / r_p² | Fabian Ramos Eq. 12–13 |

### 3. Equilibrium isotherm — Dual-Site Langmuir (Pillar A)

| Quantity | Expression | Notes |
| --- | --- | --- |
| Loading | qᵢ* = q_1,i b_i c_i /(1+Σb_j c_j) + q_2,i d_i c_i /(1+Σd_j c_j) | Fabian Ramos Eq. 14 |
| T-dependence | b_i = b_{0,i}·exp(−ΔU_{b,i}/RT); d_i = d_{0,i}·exp(−ΔU_{d,i}/RT) | Arrhenius form, Eq. 15–16 |

### 4. Energy balance (non-isothermal, Pillar A)

| Quantity | Expression | Notes |
| --- | --- | --- |
| Heat balance | [ερ_g c_{p,g}+(1−ε)ρ_p c_{p,s}]∂T/∂t + uρ_g c_{p,g}∂T/∂z = λ_eff∂²T/∂z² + (1−ε)ρ_pΣ(−ΔH_i)∂qᵢ/∂t − (4h_w/d_col)(T−T_wall) | Ramos Eq. 7; Danilov §3 |
| Isosteric heat | −ΔH_i(T,q) = −ΔU_i + RT (+ empirical loading correction) | Ramos Eqs. 9–10 |

### 5. Semi-discretised (Method of Lines) system

| Quantity | Expression | Notes |
| --- | --- | --- |
| State vector | y(t) = [c₁..c_N, q₁..q_N, T₁..T_N]ᵀ | N = number of axial nodes |
| ODE system | dy/dt = f(t,y) — spatial derivatives replaced by finite-difference / finite-volume stencils | Upwind or flux-limited for ∂/∂z per LeVeque Ch. 6–7 |
| Integration | solve_ivp(f, t_span, y0, method='BDF' or 'LSODA', jac=J, rtol≤1e-6, atol≤1e-9) | scipy docs; provide analytic/sparse Jacobian if possible |

# BDF vs LSODA — Practical Choice

|  | BDF | LSODA |
| --- | --- | --- |
| Type | Implicit, variable-order (1–5) multistep (backward differentiation) | Adams (non-stiff) / BDF (stiff) with automatic switching |
| Best when | Stiffness is known/expected up front (as here) and you can supply a Jacobian | Stiffness is uncertain or varies over the simulation (e.g. thermal front only stiff transiently) |
| Jacobian | Strongly recommended; supports sparse/callable Jacobian directly | Supported via lband/uband (banded) or finite differences; no sparse-matrix Jacobian |
| Practical note | Preferred first choice once the model is confirmed stiff, per this pillar's analysis | Convenient universal fallback; wraps legacy Fortran ODEPACK code, slightly less flexible to extend |

Recommendation for this project: start with LSODA for a quick, robust first solution while the model is still being debugged (it will auto-detect stiffness for you); switch to BDF with a supplied banded/sparse Jacobian once the discretisation is fixed and you need reproducible performance for parameter sweeps.

# Suggested Reading Sequence

One reasonable path through the pile, front-loading the conceptual foundations before the validation/edge-case papers.

| # | Card | Focus |
| --- | --- | --- |
| 1 | B1 | Ruthven Ch. 8 — governing equations, LDF derivation, front classification |
| 2 | B2 | Xu, Cai & Pan (2013) — where LDF sits among all breakthrough models |
| 3 | A2 | Al-Ghouti & Da'ana (2020) — isotherm selection & temperature dependence |
| 4 | B3 / B7 | Garg & Ruthven (1973) and Xiu (1996) — LDF vs exact diffusion benchmarks |
| 5 | B8 | Myers & Font (2020) — experimental LDF validation on CO₂ |
| 6 | A3 | Danilov et al. (2019) — closest non-isothermal analytical/numerical analogue |
| 7 | A1 | Fabian Ramos et al. (2024) — full validated CO₂/N₂ mechanism (DSL + Arrhenius + LDF + energy balance) |
| 8 | B4 / B5 / B6 | Gorius (1991), Kumar (1986), Skrip et al. (2013) — supplementary checks, read as needed |
| 9 | C1 / C2 | LeVeque Ch. 6–7 and Evans Ch. 3 — mathematical grounding for the PDE character |
| 10 | C3 | scipy solve_ivp documentation — implement the Method-of-Lines solver |

# Master Source Table

| Code | Type | Citation | Pillars |
| --- | --- | --- | --- |
| A1 | Numerical / CFD | Fabian Ramos et al. (2024), Chem. Eng. Sci. 285, 119606 | A, B, C |
| A2 | Review | Al-Ghouti & Da'ana (2020), J. Hazard. Mater. 393, 122383 | A |
| A3 | Analytical | Danilov et al. (2019), Comput. Chem. Eng. 122, 41–52 | A, C |
| B1 | Textbook | Ruthven (1984), Principles of Adsorption — Ch. 6, 8 | A, B |
| B2 | Review | Xu, Cai & Pan (2013), J. Zhejiang Univ.-Sci. A 14(3), 155–176 | B |
| B3 | Analytical | Garg & Ruthven (1973), Chem. Eng. Sci. 28(3), 791–798 | B |
| B4 | Analytical | Gorius, Bailly & Tondeur (1991), Chem. Eng. Sci. 46(2), 677–684 | B |
| B5 | Analytical | Kumar (1986), Sep. Sci. Technol. 21(10), 1039–1046 | B |
| B6 | Analytical | Skrip, Das & Chatterjee (2013), Indian Chem. Engr. 55(2), 87–103 | B |
| B7 | Analytical | Xiu (1996), Chem. Eng. Sci. 51(16), 4039–4041 | B |
| B8 | Analytical | Myers & Font (2020), Int. J. Heat Mass Transf. 163, 120374 | A, B |
| C1 | Textbook | LeVeque, Finite Volume Methods for Hyperbolic Problems — Ch. 6, 7 | C |
| C2 | Textbook | Evans, Partial Differential Equations — Ch. 3 | C |
| C3 | Technical Ref. | SciPy Developers, scipy.integrate.solve_ivp Manual (v1.18) | C |

# Full Bibliography

- Al-Ghouti, M.A. & Da'ana, D.A. (2020). Guidelines for the use and interpretation of adsorption isotherm models: A review. Journal of Hazardous Materials, 393, 122383.

- Danilov, V.A., De Schepper, P., Cousin-Saint-Remi, J. & Denayer, J.F.M. (2019). Concentration and temperature profiles in a fixed bed column based on an analytical solution of the axial dispersion model for binary and multicomponent non-isothermal adsorption processes. Computers & Chemical Engineering, 122, 41–52.

- Evans, L.C. Partial Differential Equations (2nd ed.), Chapter 3: Nonlinear First-Order PDE. American Mathematical Society, Graduate Studies in Mathematics.

- Fabian Ramos, H.S., Baliga, C., Rajendran, A. & Nikrityuk, P.A. (2024). CFD-based model of adsorption columns: Validation. Chemical Engineering Science, 285, 119606.

- Garg, D.R. & Ruthven, D.M. (1973). Theoretical prediction of breakthrough curves for molecular sieve adsorption columns — I. Asymptotic solutions. Chemical Engineering Science, 28(3), 791–798.

- Gorius, A., Bailly, M. & Tondeur, D. (1991). Perturbative solutions for non-linear fixed-bed adsorption — I. Approximate analytical solutions for asymptotic fronts. Chemical Engineering Science, 46(2), 677–684.

- Kumar, R. (1986). Column dynamics for multicomponent adsorption — constant pattern formation. Separation Science and Technology, 21(10), 1039–1046.

- LeVeque, R.J. Finite Volume Methods for Hyperbolic Problems, Chapters 6–7. Cambridge University Press.

- Myers, T.G. & Font, F. (2020). Mass transfer from a fluid flowing through a porous media. International Journal of Heat and Mass Transfer, 163, 120374.

- Ruthven, D.M. (1984). Principles of Adsorption and Adsorption Processes, Chapters 6 & 8. John Wiley & Sons, New York.

- Skrip, S., Das, G.K. & Chatterjee, S.G. (2013). Analytical expressions for the adsorbate breakthrough curve from a fixed bed of adsorbent with first-order and second-order kinetic models. Indian Chemical Engineer, 55(2), 87–103.

- Xiu, G. (1996). Modeling breakthrough curves in a fixed bed of activated carbon fiber — exact solution and parabolic approximation. Chemical Engineering Science, 51(16), 4039–4041.

- Xu, Z., Cai, J. & Pan, B. (2013). Review: Mathematically modeling fixed-bed adsorption in aqueous systems. Journal of Zhejiang University-SCIENCE A, 14(3), 155–176.

Technical reference (not a research paper): scipy.integrate.solve_ivp — SciPy v1.18 Manual

# Open Notes

General working space — for cross-cutting observations that do not belong to a single source card (e.g. discrepancies between papers, parameters you still need to source, or ideas for your own model's novel contribution).

### Session 1  —  Date: ______________

### Session 2  —  Date: ______________

### Session 3  —  Date: ______________